# Building specifications

DESIGN ARCHITECT:

S9 Architecture

ARCHITECT OF RECORD

Perkins Eastman

MECHNICAL ENGINEER:

Cosentini Associates

Structural engineer:

Arup

landscape ARCHITECT:

MPFP

Building size:

675,000 RSF

floors:

17 total

Floors 1-2: amenities

Floors 3-15: office

Floor 16: amenity

Floor 17: mechanical

Typical floor:

High Rise: 42,000 RSF average

Mid Rise:  50,000 RSF average

Low Rise:  33,000 RSF

building Module:

5’0”

OFFICE Floor heights:

14’0” Slab-Slab

OFFICE  FLOOR COLUMN SPACING:

30’0” Typical

Electrical:

8 watts/USF for lighting and convenience power

Elevators:

West, Center and East Core with 12 destination dispatch elevators

Capacity: 3,500 LB

Freight: 1 swing car in the East Core at 4,500 LB

Speed:  500 FPM (East and West Core), 350 FPM (Center Core)

HVAC:

Condenser water/Cooling towers, Local floor, Tenant controlled DX units

Heating provided overhead

Up to 1 ton/1500 USF of supplemental capacity available to tenants at additional cost

Structure:

Steel frame with steel braced cores

Floor system of concrete fill on metal deck over steel beams

Parking:

Dedicated parking lot of 122 stalls

Security:

Brooklyn Navy Yard 24/7 perimeter vehicular and pedestrian security

24-7 attended lobby

CCTV monitoring of common areas, elevators and loading dock

Fire and Safety:

Four egress stairs, dedicated emergency service elevator

1,500 kw generator for life safety, 500 kw capacity for tenant tie-in available to tenants at additional cost

Floor loading:

Typical 50 lbs/PSF – Live load.  Localized increases

LEED:

Target LEED Silver

AMENITIES:

18,000 SF of private outdoor terraces on 6 floors

35,000 SF of common indoor amenities: food & beverage, health & wellness, conference facilities

10,000 SF of common outdoor amenities: lawn, sun deck, ½ basketball court